HARWINTON PAVING SCOPE OF SERVICES

Harwinton Paving shall clean the existing asphalt road surface of all foreign and loose materials by blowing cracks clean using air compressor and wand per standard practices. All new cracks, any previously sealed cracks that have “opened” to a width of ¼” and a depth of ¼” below the surface of the surrounding pavement, and exhibiting separation from the adjacent pavement shall be sealed, or resealed. When cracks show evidence of vegetation, the vegetation shall be removed, using a powered wire wheel grazing tool, or torch as needed to eliminate all vegetation, dirt, moisture and seeds.  No crack-sealing material shall be applied in wet cracks, or, where frost, snow or ice is present. The pavement temperature shall be above 40 degrees Fahrenheit at the time of the installation of the crack sealing material. A heat lance shall be used to bring pavement temperature to the listed temperature of 40 degrees and to dry any moisture.

The prices listed in the schedule include Certified Flaggers for traffic control. If Uniformed Police are requested by the Municipality, said Uniformed Police shall be paid directly by the Municipality thus eliminating any delay in payment or administration fee.