SCOPE OF SERVICES

Northeast Crack Filling Inc. shall clean the existing asphalt road surface of all dirt, sand, oil, grease and loose materials by blowing cracks clean using on board air compressor and wand. All unsealed cracks, all previously sealed cracks with the surface of the existing sealant greater than or equal to ¼” below the surface of the surrounding pavement, and all previously sealed cracks exhibiting cracks, holes, voids and separation from the adjacent pavement shall be sealed. Immediately prior to sealing, all cracks shall be thoroughly cleaned of all foreign material. When cracks show evidence of vegetation, the vegetation shall be removed, using a powered wire wheel grazing tool, eliminating all vegetation, dirt, moisture and seeds.  No crack-sealing material shall be applied in wet cracks, or, where frost, snow or ice is present. The pavement temperature shall be above 40 degrees Fahrenheit (4 degrees Celsius) at the time of the installation of the poured sealing material. A heat lance shall be used to bring pavement temperature to the listed temperature of 40 degrees and to dry any moisture.

The prices listed in the schedule include Certified Flaggers for traffic control. If Uniformed Police are requested by Municipality, Northeast Crack Filling Inc. shall be reimbursed the cost of the uniformed officers upon providing said Municipality a receipt that the invoices for said officers has been paid. No Administration fee will be applied to this portion of the work.